LOT 2

TECHNICAL SPECIFICATION NO. 6

NAME: WEATHER STATION

QUANTITY: List with the quantities of tools and technological equipment, including endowments

MODEL/YEAR OF MANUFACTURE: 2022-2023

MANUFACTURER/COUNTRY:

| MINIMUM TECHNICAL CHARACTERISTICS | YES | NO | OBSERVATIONS |
| --- | --- | --- | --- |
| 1.WEATHER STATION CONNECTION BOX |  |  |  |
| Power supply:100-230V; 16A AC |  |  |  |
| Dimension(mm): maximum 800x800x350 |  |  |  |
| Degree of protection: IP65 or equivalent |  |  |  |
| Heating/cooling system(W): minimum 100 |  |  |  |
| Power supply: 24V 20A |  |  |  |
| UPS 24V, min 7Ah |  |  |  |
| Ethernet ports: minimum 5 |  |  |  |
| Integrated F-type socket: Yes |  |  |  |
| Operating temperature: -200C si 550C |  |  |  |
| 2. WEATHER STATION FOR PERFORMANCE RATIO MEASUREMENT - PR |  |  |  |
| 2.1 DATA LOGGER COMPATIBLE AND EQUIPPED FOR SENSOR CONNECTION - INCLUDED |  |  |  |
| 2.2. PYRANOMETER |  |  |  |
| Spectral range (nm): 285-2800 |  |  |  |
| Response time(s): maximum 2 |  |  |  |
| Thermal radiation at 200 W/m²: maximum 7 W/m² |  |  |  |
| Temperature change at 5 K/h: maximum 2 W/m² |  |  |  |
| Power consumption at 12 V DC: maximum 150 mW |  |  |  |
| Operating temperature: -200C and 800C |  |  |  |
| Degree of protection: IP67 or equivalent |  |  |  |
| Software, Windows or equivalent: possibility for configuration, testing and data logging |  |  |  |
| 2.3. COMPONENT: AMBIENT SENSOR - INCLUDED |  |  |  |
| 2.4. COMPONENT: COMPACT WEATHER STATION - INCLUDED |  |  |  |
| Parameters measured: Temperature, Relative Humidity, Precipitation intensity, amount and type, Pressure, Wind direction and speed. |  |  |  |
| Own consumption: maximum 50VA |  |  |  |
| Degree of protection: IP66 or equivalent |  |  |  |
| Standards: IEC 61724-1:2017 or equivalent |  |  |  |
| 2.5 COMPONENT: PANEL SENSOR - INCLUDED |  |  |  |
| 3. OTHER REQUIREMENTS |  |  |  |
| 3.1. MECHANICAL CLAMPING SYSTEMS - INCLUDED |  |  |  |
| 3.2. FULLY EQUIPPED POWER SUPPLY PANEL - INCLUDED |  |  |  |
| OTHER REQUIREMENTS Documents to be submitted by the contractor upon delivery of the products: i. Declaration of conformity ii. Warranty certificate iii. List of delivered components iv. Commissioning/Programming software line All training materials and manuals shall be written in Romanian or English and shall contain all information necessary for the operation and maintenance of the equipment by the beneficiary's authorised personnel |  |  |  |
| CERTIFICATION |  |  |  |
| The equipment must meet all legal conditions for placing on the market in Romania by presenting the European CE Mark certificate and or the manufacturer's Declaration of Conformity with: - SR EN 62271-200 or equivalent, SR EN 62271-102 or equivalent, SR EN 62271-202 or equivalent, IEC 466 or equivalent, IEC 265/1-83 or equivalent, IEC 466/87 or equivalent. |  |  |  |
| WARRANTY AND POST WARRANTY |  |  |  |
| Warranty period: minimum 60 months from the date of signature of the Equipment Acceptance Certificate. Possibility of warranty extension. |  |  |  |
| ASSOCIATED SERVICES INCLUDED |  |  |  |
| Transport to be carried out at the delivery location indicated by the beneficiary. These operations will be included in the final price of the equipment |  |  |  |